# Door Lock Control ECU Component Requirements Specification (Lastenheft)

# Document Information

Title: Component Requirements Specification for Door Lock Control ECU

Project: Automotive ECU Development Demo

Version: 1.1 (Merged)

Date: September 27, 2025

Customer: Volkswagen Group (simulated)

Supplier: To be assigned

Compliance: Automotive SPICE 4.0 (SYS.1 Requirements Elicitation), ISO 26262:2018 (ASIL A), ISO/SAE 21434, VDA KLH Gelbband 2023, Volkswagen Formel Q, IATF 16949

This merged document integrates the original specification with the provided draft, eliminating redundancies (e.g., duplicate safety/compliance items) and resolving discrepancies (e.g., harmonizing dimensions: 100x80x30mm vs. 100x70x30mm to stricter 100x70x30mm; weights: 200g vs. 150g to 150g; power: 1W vs. 50mA to 50mA/1W equivalent). Requirements are precise, traceable, and at SYS.1 level for input to SYS.2–SYS.5, SWE.1–SWE.6. Total: >50 requirements.

# Non-binding Recommendation

Compliant with VDA guidelines. Implementation at user's risk. Copyright protected.

# Foreword

Defines requirements for Door Lock Control ECU to control locks via key fob/RF (125kHz wake-up, 433MHz command), buttons, with status feedback. Incorporates agile iteration, aligns with VDA MLA, addresses software, safety (ASIL A), cybersecurity. Modular structure: Module I (process), Module II (product).

# Contents

Introduction

Objective and Purpose

General Description and Definitions

Application

Formulating Precise Requirements

Requirements Management Process

Module I – Process and Organization Originated Requirements

Module II – Project and Product-Specific Requirements

Definitions, Terms, Abbreviations

# 1. Introduction

ECU manages door locking/unlocking with binary I/O, FSM logic, <500ms response, fail-safe unlock. Challenges: Safety, security, compliance in supply chain. Bundles requirements for series development.

# 2. Objective and Purpose

2.1 Significance
Unambiguous product profile.

2.2 Objective
Standardize interfaces; reduce errors.

2.2.1 Added Value for Customer
Meets Volkswagen standards, eases integration.

2.2.2 Added Value for Suppliers
Clear inputs, risk mitigation.

# 3. General Description and Definitions

3.1 General Description
System-level ECU with hardware/software/interfaces, operates across variants/environments.

3.2 Definitions
CRS (Lastenheft): Customer requirements.
Functional Specification (Pflichtenheft): Supplier response.
See Section 9 for others.

# 4. Application

4.1 General Information
Applies in supply chain; modular.

4.2 Overview of Modules
4.2.1 Module I
Process requirements (e.g., management, safety).

4.2.2 Module II
Product-specific (e.g., functions, compliance).

# 5. Formulating Precise Requirements

5.1 Objective
Unambiguity, traceability.

5.2 Quality Criteria
Atomic, verifiable, prioritized (shall/must); avoid weak words.

# 6. Requirements Management Process

Iterative per ASPICE SYS.1: Elicit, analyze, document, validate using tools (e.g., Doors NG). Supply chain: Lastenheft to supplier; Pflichtenheft response; joint reviews. Feasibility analysis within 4 weeks post-award.

# 7. Module I – Process and Organization Originated Requirements

Compliant with IATF 16949, Formel Q; follows ASPICE lifecycle, VDA MLA milestones.

## 7.1 Project Management

REQ-I-1.1: Supplier shall establish ASPICE MAN.3 project plan, aligned with customer SOP and VDA MLA.

REQ-I-1.2: Supplier shall conduct bi-weekly reviews and milestone reports per Formel Q.

REQ-I-1.3: Supplier shall manage resources, including software tools.

## 7.2 Risk Management

REQ-I-2.1: Supplier shall perform risk analysis (FMEA, HARA ASIL A, TARA) per ISO 26262 Part 3 and ISO 21434 Clause 8.

## 7.3 Environment and Sustainability

REQ-I-3.1: Supplier shall comply with REACH, RoHS, minimize production energy.

## 7.4 Functional Safety

REQ-I-4.1: Supplier shall develop per ISO 26262 ASIL A, provide safety case.

## 7.5 Cybersecurity Management

REQ-I-5.1: Supplier shall implement ISO 21434 and UNECE R155 processes.

## 7.6 Software Update Management

REQ-I-6.1: Supplier shall support OTA per UNECE R156.

## 7.7 Data Protection

REQ-I-7.1: Supplier shall comply with GDPR and TISAX.

## 7.8 Supply Chain Management

REQ-I-8.1: Supplier shall audit sub-suppliers per VDA 6.3, ensure traceability per IATF 16949 8.5.2.

## 7.9 Releases

REQ-I-9.1: Supplier shall obtain PPAP per VDA Volume 2.

## 7.10 Quality Management System

REQ-I-10.1: Supplier shall maintain IATF 16949 certification, annual self-audits per Volkswagen CSRs.

## 7.11 Quality Assurance

REQ-I-11.1: Supplier shall implement process FMEA.

## 7.12 Requalification

REQ-I-12.1: Supplier shall requalify every 3 years per Volkswagen CSRs.

## 7.13 Reporting and Documentation

REQ-I-13.1: Supplier shall report via VW B2B platform, bi-weekly status.

## 7.14 Spare Part and Service Requirements

REQ-I-14.1: Supplier shall ensure 15-year spares.

## 7.15 Configuration and Variant Management

REQ-I-15.1: Supplier shall manage variants per ASPICE ACM.2.

## 7.16 Non-Conformity Management

REQ-I-16.1: Supplier shall handle 8D reports.

## 7.17 Change Management

REQ-I-17.1: Supplier shall notify changes per VW 01155.

## 7.18 Data Management

REQ-I-18.1: Supplier shall use IMDS.

## 7.19 Test Management

REQ-I-19.1: Supplier shall define tests per ISO 26262 Part 8.

## 7.20 Standard and Carry-Over Parts

REQ-I-20.1: Supplier shall use approved carry-overs.

## 7.21 Logistics

REQ-I-21.1: Supplier shall comply with VW packaging.

## 7.22 Labeling and Traceability

REQ-I-22.1: Supplier shall label per VW 34022, provide traceability (Doors NG equivalent).

## 7.23 Requirements Regarding Test and Prototype Tools

REQ-I-23.1: Supplier shall provide prototypes per MLA.

## 7.24 Applicable Documents

REQ-I-24.1: Reference Formel Q, ISO 26262, ISO 21434.

# 8. Module II – Project and Product-Specific Requirements

## 8.1 Objective and Scope of the Project

REQ-II-1.1: ECU shall control 4 doors (scalable to variants), including hardware/software/integration.

REQ-II-1.2: Scope: Lock/unlock via key fob RF (125kHz wake-up, 433MHz), buttons, diagnostics; fail-safe unlock on fault/power loss.

## 8.2 Traceability

REQ-II-2.1: All requirements shall trace to tests/verification.

REQ-II-2.2: Traceability matrix shall be provided.

## 8.3 Description of the Product and Function

REQ-II-3.1: ECU shall use FSM (Locked, Unlocked, Transition) for binary control.

REQ-II-3.2: Inputs: Key fob, door buttons, diagnostic; Outputs: Actuators (200–500ms pulse), status to body controller via CAN.

## 8.4 Product-Specific Requirements

REQ-II-4.1: ECU shall operate -40°C to +85°C, humidity 95% RH.

REQ-II-4.2: Power: 9–16V (jump 24V), <50mA idle/<2A peak.

## 8.5 Technical and Functional Requirements

REQ-II-5.1: ECU shall process lock/unlock within ≤500ms.

REQ-II-5.2: ECU shall activate actuators with priority unlock.

REQ-II-5.3: ECU shall detect jams, retry 3 times.

REQ-II-5.4: ECU shall use CAN FD 500kbps per SAE J1939.

REQ-II-5.5: ECU shall authenticate key fob (rolling code).

REQ-II-5.6: ECU shall unlock on crash signal.

REQ-II-5.7: ECU shall enter low-power <9V.

REQ-II-5.8: ECU shall support UDS diagnostics (ISO 14229).

REQ-II-5.9: Software shall debounce 50–100ms, ignore duplicates.

REQ-II-5.10: ECU shall handle 4 actuators independently.

REQ-II-5.11: ECU shall support child lock via CAN/central switch.

REQ-II-5.12: ECU shall report status to gateway.

REQ-II-5.13: Fail-safe: Unlock on power loss/watchdog reset.

REQ-II-5.14: ECU shall resist EMI per ISO 11452.

REQ-II-5.15: Hardware: Microcontroller ≥32KB flash, H-bridge 12V/3A.

REQ-II-5.16: Software shall use watchdog.

REQ-II-5.17: ECU shall protect overcurrent >3A, stop <50ms.

REQ-II-5.18: Integrate with keyless entry.

REQ-II-5.19: ECU shall log faults/DTCs to non-volatile memory (ISO 15031).

REQ-II-5.20: ECU shall ignore invalid, protect replay attacks.

REQ-II-5.21: ECU shall support LIN for variants.

REQ-II-5.22: ECU shall provide anti-bounce for switches.

REQ-II-5.23: ECU shall detect stuck relay, signal DTC.

REQ-II-5.24: ECU shall support seed/key access.

REQ-II-5.25: ECU shall report SW/HW version, VIN.

REQ-II-5.26: ECU shall support actuator tests via UDS.

REQ-II-5.27: ECU shall provide snapshot on DTC.

REQ-II-5.28: ECU shall support secure boot.

REQ-II-5.29: ECU shall encrypt OTA.

REQ-II-5.30: ECU shall implement intrusion detection.

## 8.6 Product Compliance

REQ-II-6.1: Comply with ECE R10 EMC (CISPR 25 Class 3).

REQ-II-6.2: Materials: ELV, RoHS, REACH.

REQ-II-6.3: ASIL A per ISO 26262.

REQ-II-6.4: Cybersecurity per ISO 21434 (UNECE R155).

## 8.7 Specific Test Requirements

REQ-II-7.1: Functional tests: 100% state coverage.

REQ-II-7.2: Endurance: 100,000 cycles, 10g vibration (ISO 16750), load dump (ISO 7637-2).

REQ-II-7.3: Safety validation per ISO 26262 Part 9.

REQ-II-7.4: Penetration testing.

## 8.8 Non-Functional Requirements

REQ-II-8.1: MTBF ≥15 years/250,000km.

REQ-II-8.2: Weight ≤150g.

REQ-II-8.3: Dimensions ≤100x70x30mm.

REQ-II-8.4: Power <50mA idle.

REQ-II-8.5: IP67 protection.

REQ-II-8.6: Vibration per ISO 16750-3.

REQ-II-8.7: Maintainability: MISRA-C 2012.

REQ-II-8.8: Scalable to 2-door.

REQ-II-8.9: Cost <€20/unit.

REQ-II-8.10: Volume 100,000/year.

REQ-II-8.11: Lifecycle 15 years.

REQ-II-8.12: Recyclable >95%.

REQ-II-8.13: Restart ≤200ms.

REQ-II-8.14: Availability ≥99.9%.

REQ-II-8.15: Reverse polarity -14V.

# 9. Definitions, Terms, Abbreviations

ECU: Electronic Control Unit
FSM: Finite State Machine
ASIL: Automotive Safety Integrity Level
TARA: Threat Analysis and Risk Assessment
DTC: Diagnostic Trouble Code
UDS: Unified Diagnostic Services
Weak word list: Avoid "should"; use "shall".